Safe Work Procedure

NOTE: DO NOT use this machine unless you have been trained in its safe use and operation.

| Description of Work: |  |  | Using a Hydraulic Press |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Potential Hazards: Potential to cause harm as a result of crushing from moving parts, hazardous manual handling, materials being ejected from the plant and chemical hazard (release of hydraulic fluid). |  |  |  |  |  |
| Personal Protective Equipment (PPE) Required (Check the box for required PPE): |  |  |  |  |  |  |  |  |  |  |  |
| Gloves |  | Face Masks |  | #### Eye #### Protection | Welding Mask |  | Appropriate Footwear |  | Hearing Protection | Protective Clothing |  |
| Safe Work Procedure Checklist: |  |  |  |  |  |  |  |  |  |  |  |
| 1. PRE-Operation/Task: - Task (e.g. Drawings, instructions, specifications etc.) is clearly understood. - Ensure press is in good working order without visible damage to structure or hydraulic fluid leaks. - Ensure work piece is placed squarely on the plate. - Clamp work piece (if appropriate). 2. Operation/Task: - Keep hands and body clear of press mechanism when lowering. - Lower ram until it contacts the work piece. - Adjust pressure of ram to desired setting. - Identify ON/OFF switch and emergency stop button (if applicable). 3. POST-Operation/Task: - Ensure the area is clear of other people. - Release pressure on hydraulic press. - Slowly raise ram and remove work piece from plate. - Return work pieces and other items to storage area as appropriate. |  |  |  |  |  |  |  |  |  |  |  |
| Competent Person(s): (The following persons are authorised to operate, supervise and test students on the equipment/process). |  |  |  |  |  |  |  |  |  |  |
| Name: | Title: |  |  |  |  |  |  | Contact Details: |  |  |