Safe Work Procedure

NOTE: DO NOT use this machine unless you have been trained in its safe use and operation

| Description of Work: |  |  | Using a Power Hacksaw |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Potential Hazards: Exposed moving parts and electrical hazard with the potential to cause harm through entanglement, impact and cutting, exposure to heat, noise, projectiles, sharp objects, friction and sparks. |  |  |  |  |  |
| Personal Protective Equipment (PPE) Required (Check the box for required PPE): |  |  |  |  |  |  |  |  |  |  |  |
| Gloves |  | Face Masks |  | #### Eye #### Protection | Welding Mask |  | Appropriate Footwear |  | Hearing Protection | Protective Clothing |  |
| Safe Work Procedure Checklist: |  |  |  |  |  |  |  |  |  |  |  |
| 1. PRE-Operation: - Task (e.g. Drawings, instructions, specifications etc.) is clearly understood. - Identify ON/OFF switch and emergency stop button (if applicable). - Ensure that work is securely clamped in the work vice. - Check coolant delivery system to allow for sufficient flow of coolant. - Faulty equipment must not be used. 2. Operation: - Ensure that work piece is secure. - Support overhanging work and use appropriate signage if it presents a hazard. - Keep clear of moving machine parts. - Direct coolant onto blade before starting cutting. - Never leave the machine running unattended. - Before making adjustments or before cleaning waste material accumulations, switch off and bring machine to a complete standstill. 3. POST-Operation: - Switch off machine and store equipment machine in a safe and clean state. - Ensure good housekeeping practices are in place to minimise dust build-up. |  |  |  |  |  |  |  |  |  |  |  |
| Competent Persons (The following persons are authorised to operate, supervise and test students on the equipment/process). |  |  |  |  |  |  |  |  |  |  |
| Name: | Title: |  |  |  |  |  |  | Contact Details: |  |  |